# LoKy Tutorial

# Arduino IDE:

## Setup board information:

## Install version 1.6.21 on Boards Manager (the higher versions have bugs for LMiC library)

  - Select Processor: “ATmega328P (3.3V, 8MHz)” and Port that is connecting to ProMini

  - Install Libraries following this instruction: https://www.arduino.cc/en/guide/libraries

# The Things Network (TTN)

- Login to TTN console: https://console.thethingsnetwork.org/ and add a new Application

- Device register

- After registering an OTAA device, adjust the keys into the following format

# Payload decoder on TTN

- Change the decoder to the LoKy_decoder and remember to SAVE before add integration

# Integrations on TTN

- Add Data Storage integration:

- Now, we have everything to run:

- LoKy main board

- Arduino and its stuff

- last but not least, TTN configured.

# LoKy Firmware upload:

      - First we need to replace the keys (in section 2.c) from TTN in “3_Device_Keys.h”

      - Depending on the “option tarifaire” of Linky, we can use BASE or HCHP for our device by comment and uncomment line 11 or 12 of the main sketch.

      - Upload the sketch and check the operation of device from the Serial Monitor on Arduino IDE.

      - Finally, we can plug LoKy to the compteur and done!

- Section 6 is used for Application server and Database.

# Application Server

- Install InfluxDB and Grafana following this instruction:

https://github.com/ITU-PITLab/public/blob/master/TheThingsNetwork%2Bnode-red%2Binfluxdb%2Bgrafana.md

- Install Node-RED

If Git is not installed on your PC: https://git-scm.com/downloads. After that, run this command on Gitbash   sudo npm install -g --unsafe-perm node-red

*Credit to GitHub: Fabien FERRERO – UCA board